5th edition Goljan Rapid Review, page 17

Definition: Pulse oximetry (Fig. 2-1 A) is a noninvasive test for measuring SaO2. It utilizes a probe that is usually clipped over a patient’s finger. A pulse oximeter emits light at specified wavelengths that identify oxyhemoglobin (over the range of 100% to ≈75%) and deoxyhemoglobin, respectively. The wavelengths emitted by a pulse oximeter cannotidentify dyshemoglobins such as methemoglobin (metHb; heme group has +3 valence) and carboxyhemoglobin (i.e., carbon monoxide bound to Hb [COHb]), which normally decrease the SaO2 (see later). In the presence of these dyshemoglobins, the oximeter calculates a falsely high SaO2 (Fig. 2-1 B). However, unlike the standard oximeter, a co-oximeter emits multiple wavelengths and identifies metHb and COHb as well as oxyhemoglobin and deoxyhemoglobin. Hence, in the presence of these dyshemoglobins, the SaO2 will be decreased. Pulse oximeters are very useful in following patients with respiratory failure, severe asthma, obstructive sleep apnea, and those under general anesthesia. Most clinicians consider pulse oximeters to be inaccurate when SaO2 values are less than 70%